Low Water Use Landscaping Requirements Tariff for Model Homes in New Residential Developments – BMP 5.5

PURPOSE

A program for the Company to reduce water use within its service area and/or increase water use efficiency by limiting or reducing water used for specific purposes (Modified Non-Per Capita Conservation Program BMP Category 5: Ordinances/Conditions of Service/Tariffs 5.5: Low Water Use Landscaping Requirements Tariff for Model Homes in New Residential Developments).

REQUIREMENTS:

The requirements of this tariff are governed by Rules of the Arizona Corporation Commission, specifically A.A.C. R14-2-403 and R14-2-410 and were adapted from the Arizona Department of Water Resources’ Required Public Education Program and Best Management Practices  in the Modified Non-Per Capita Conservation Program.

- The Company shall provide to the customer/developer/builder distinct guidelines for landscape planning and design for model homes.

- The following landscape restrictions will be required in order for a customer/developer/builder to receive water service to its model homes from the Company on or after the effective date of this tariff:

Restrictions Applicable to All New Model Home Landscaping -

  - Model home landscaping will involve strategic planning and design. Landscaped areas will be divided into zones based on water requirements. Each model will have efficient irrigation systems, properly designed and maintained.

  - All models will be placed on a drip irrigation system with a timer for shrubs and trees to apply water directly to the roots where it is needed. Watering schedules will be adjusted each month to match seasonal weather conditions and landscape requirements.

  - All front yards shall be landscaped with xeriscape (low water use) materials.  A list of low water use landscaping materials is available from the Company upon request.  No turf of any kind that requires watering shall be allowed in front yards.

  - Turf in back yards shall be limited to no more than fifty percent (50%) of the total backyard area.

  - No model home shall be equipped with a swimming pool, jacuzzi, or other water-use intensive feature (e.g., fountain, fish pond, etc.).

  - Model home landscapes will require minimal but appropriate maintenance. Landscape maintenance may include pruning, removing trash that has blown into the landscape, occasional weeding and pest management, checking that the irrigation system is functioning properly, and adjusting automatic irrigation systems as the seasons change.

3.  Subject to the provisions of this tariff, the installation of the landscape restrictions will be a condition of service.

4.	The Company shall provide to customer/developer/builder a complete copy of this tariff and all attachments upon request for service.  The customer/developer/builder shall follow and abide by these landscape restrictions.

5. If after a customer has been connected to the Company water system, the Company discovers that the customer has installed turf or water-use intensive features contrary to the above requirements, the Company shall notify (in writing) the customer of such violation and provide the customer with the appropriate educational materials informing the customer of some possibilities of how to correct the problem.  The customer shall be allowed thirty (30) days to come into compliance with the above requirements.  If after  thirty (30) days the customer is not in compliance with the above requirements, the customer’s service may be terminated per Arizona Administrative Code R14-2-410C, R14-2-410D and R14-2-410E.

6.  If a customer believes he/she has been disconnected in error, the customer may contact the Commission's Consumer Services Section at 1-800-222-7000 to initiate an investigation.